TeslaBMS M5Dial

Operating Manual  •  v6

ESP32-S3 BMS Monitor for Tesla & BMW i3 Battery Modules. Ported from collin80/TeslaBMS; Tom-evnut/BMSV2; ronaegis/tesla-bms-esp32s3

# 1. Overview

TeslaBMS M5Dial is a battery management monitor and controller running on the M5Stack M5Dial (ESP32-S3). It supports two slave CMU types and communicates over a 240×240 round display with a rotary encoder for navigation.

| Feature | Detail |
| --- | --- |
| CMU support | Tesla BQ76PL536A UART • BMW i3 CSC CAN bus |
| Max modules | 20 Tesla / 8 BMW i3 (12 cells each) |
| Display | 240×240 GC9A01 round, LVGL 8.x |
| CAN | SimpBMS-compatible TX + RX (Grove B + transceiver) |
| WiFi | Access Point, dashboard at 192.168.4.1 |
| Active balancing | Tesla UART mode only; inhibited in drive mode |
| EEPROM version | 0x17 (v6) — auto-resets to defaults on mismatch |

# 2. Hardware

## 2.1 Required Hardware

- M5Stack M5Dial (ESP32-S3FN8, 240×240 GC9A01 round display)

- Tesla CMU module(s) — OR — BMW i3 CSC modules with CAN transceiver

- SN65HVD230 or MCP2551 CAN transceiver (Grove B, for CAN output or i3 mode)

- Grove 4-pin cable(s)

## 2.2 GPIO Pin Assignment

| Function | GPIO | Connector | Notes |
| --- | --- | --- | --- |
| CMU UART RX | 13 | Grove A Yellow | Tesla mode only |
| CMU UART TX | 15 | Grove A White | Tesla mode only |
| CAN TX (TWAI) | 1 | Grove B Yellow | Requires CAN transceiver |
| CAN RX (TWAI) | 2 | Grove B White | Requires CAN transceiver |
| Drive inhibit | 38 | Expansion pad | INPUT_PULLDOWN; HIGH = inhibit balancing |
| Encoder A/B/Btn | 40/41/42 | Built-in | Rotary navigation |
| Power latch | 46 | Built-in | Must be set HIGH immediately on boot |

## 2.3 Tesla CMU Wiring — Grove Port A

| CMU Pin | Wire | Grove A Pin | Direction |
| --- | --- | --- | --- |
| Pin 5 (5V) | Red | Pin 2 (5V) | Power to CMU |
| Pin 3 (GND) | Green | Pin 1 (GND) | Common ground |
| Pin 4 (TX) | Yellow | Pin 3 (GPIO13) | CMU TX → M5Dial RX |
| Pin 2 (RX) | Blue | Pin 4 (GPIO15) | M5Dial TX → CMU RX |

Note: No voltage divider required. CMU signals are 3.3V compatible.

Multiple modules daisy-chain via their second connector. Address assignment is automatic on boot.

## 2.4 CAN Transceiver Wiring — Grove Port B

Required for CAN output (SimpBMS frames) and BMW i3 CAN slave mode. Connect an SN65HVD230 or MCP2551 module:

| Grove B Pin | Transceiver Pin | Notes |
| --- | --- | --- |
| Pin 1 (GND) | GND |  |
| Pin 2 (5V) | VCC | 3.3V transceivers also work |
| Pin 3 (GPIO1) | TXD | CAN TX |
| Pin 4 (GPIO2) | RXD | CAN RX |

Note: Grove Port A (GPIO13/GPIO15) is reserved for Tesla UART. Do not connect the transceiver there.

# 3. CMU Mode Selection

v6 supports two CMU (Cell Monitoring Unit) types. The mode is stored in flash and survives reboots. Change via the web dashboard Settings tab, or with the serial console command CMUTYPE=.

## 3.1 Tesla UART Mode (default, CMUTYPE=0)

- BQ76PL536A daisy-chain protocol at 612,500 baud on Grove A

- Supports up to 20 modules, 4/5/6 cells each (global or per-module)

- Active cell balancing via register writes (BAL_CTRL, BAL_TIME)

- On boot: broadcasts wake, renumbers addresses, scans for all modules

## 3.2 BMW i3 CAN Mode (CMUTYPE=1)

- BMW i3 CSC modules broadcast autonomously once woken over CAN

- Master sends a single wake frame (0x130, 8 zero bytes) at startup

- Cell voltage frames: 0x3D1–0x3D8 (3 sub-frames per module, 4 cells each = 12 cells/module)

- Temperature frames: 0x3B1–0x3B8 (two sensors per module, 0.1°C/bit signed)

- 12 cells per module — Grove A UART is not used in this mode

- Balancing: BMW i3 CSC modules balance internally — no commands sent by the BMS

Note: A CAN transceiver on Grove B is required for BMW i3 mode.

# 4. Display Interface

## 4.1 Pack Summary Page (Page 0)

Shown at boot and whenever the encoder button is short-pressed. Displays:

- Pack voltage — large, white. Turns red if any module is faulted

- State of Charge % — blue, linear estimate from configurable SoC Lo/Hi voltage range

- Lowest and highest cell voltages across all modules

- Cell delta (dV:) — highest minus lowest. Turns amber if delta > 50mV

- Average pack temperature

- Module count

- WiFi IP address (or 'WiFi: off' when disabled)

- Green arc = healthy, Red arc = faulted

## 4.2 Module Detail Pages

Rotate the encoder to navigate through detected modules. Each module page shows:

- Module number and total module voltage in the title

- All cell voltages in large (montserrat-20) text — up to 6 cells for Tesla, 12 for BMW i3

- Colour-coded bar graphs: green = normal, red = below UV threshold, amber = above OV threshold

- Both temperature sensor readings (T1, T2)

- Fault badge: green OK or red FAULT:0xXX with register value

Note: Inactive cell slots (beyond the configured cell count) are automatically hidden.

## 4.3 Settings Page

Accessible by long-pressing the encoder button for more than 1 second, then rotating to the settings page. Displays current operating parameters:

- CMU type (Tesla UART or BMW i3 CAN)

- Cells per module, series and parallel string count

- Overvoltage and undervoltage thresholds

- Balance voltage and hysteresis

- BalInh: GPIO (HIGH = drive inhibit active) and CAN (charger heartbeat status)

- Charger heartbeat CAN ID and enable/disable state

- Auto-balance on/off and detected module count

## 4.4 Encoder Controls

| Action | Result |
| --- | --- |
| Rotate clockwise | Next module detail page |
| Rotate counter-clockwise | Previous module detail page |
| Short press (< 1 s) | Return to pack summary (page 0) |
| Long press (≥ 1 s) | Toggle WiFi on/off at runtime |

# 5. WiFi Dashboard

## 5.1 Access

WiFi operates in Access Point mode. SSID: TeslaBMS  Password: teslapack

Connect a device to the TeslaBMS network, then browse to: http://192.168.4.1

WiFi can be enabled at boot (Settings tab or WIFI=1 serial command) or toggled at runtime with a long encoder press.

## 5.2 Dashboard Tabs

| Tab | Content |
| --- | --- |
| Pack & Modules | Live pack voltage, SoC, cell lo/hi/delta, temperature. Per-module cards with individual cell voltage bars. |
| CAN Feed | Rolling log of last 50 CAN frames with decoded labels. Manual CAN frame transmit tool. |
| Settings | All thresholds, CMU type, CAN inhibit mode, charger heartbeat ID, per-module cell count overrides. Changes saved instantly to flash. |

# 6. CAN Bus

## 6.1 Outbound SimpBMS Frames (TX)

Transmitted every 1 second at 500 kbps. Compatible with Victron, SMA, and other SimpBMS-supporting inverters/chargers.

| CAN ID | Content |
| --- | --- |
| 0x351 | Charge voltage limit, charge current limit, discharge current limit, discharge voltage limit (0.1V/A per bit, little-endian) |
| 0x355 | State of Charge % (1%/bit) and State of Health (fixed 100%) |
| 0x356 | Pack voltage (0.01V/bit), current from CAB300 sensor (0.1A/bit signed), average temperature (0.1°C/bit) |
| 0x35A | Alarm and fault bitmask (OV, UV, OT flags) |
| 0x35E | Manufacturer string 'TeslaBMS' |
| 0x35F | Chemistry 'LI', hardware v1.0, firmware v6.0 |

## 6.2 Inbound CAN (RX) — v6

A dedicated FreeRTOS task on core 0 receives all incoming CAN frames. Received frames are used for two purposes:

- Charger/inverter heartbeat detection — any frame matching the configured Charger HB ID (default 0x305, CAB300 current sensor) resets a 5-second timeout. Current from 0x305/0x306 is forwarded to the 0x356 TX frame.

- BMW i3 CSC cell voltage and temperature decoding (0x3D1–0x3D8, 0x3B1–0x3B8)

## 6.3 Balance Inhibit via CAN

When CAN Balance Inhibit is enabled (CANINHIBIT=1), balancing is paused if no charger heartbeat frame is received for more than 5 seconds. This prevents balancing during driving when the charger is absent.

The inhibit source is displayed on both the dial Settings page (BalInh: GPIO=... CAN=...) and the web dashboard.

| CAN Status | Meaning |
| --- | --- |
| CAN=off | CAN inhibit disabled — GPIO38 only |
| CAN=CHG-OK | Charger heartbeat seen recently — balancing allowed |
| CAN=TIMEOUT | No heartbeat for >5 s — balancing inhibited |

# 7. Active Balancing (Tesla UART Mode Only)

Balancing is disabled by default. Enable via AUTOBAL=1 serial command or the web Settings tab.

## 7.1 Balancing Logic

- Each cell that is above the pack's lowest cell voltage AND above the BALVOLT threshold has its BAL_CTRL bit set

- A 60-second hardware timer in the BQ76PL536A limits each balance burst

- Balancing repeats every 1 second while conditions are met

## 7.2 Inhibit Sources

- GPIO38 HIGH — vehicle drive signal. Wire to a 3.3V or 5V-divided drive signal.

- CAN heartbeat timeout — if CANINHIBIT=1 and no charger frame seen for 5 s.

- Pack fault active — any module reporting a non-CUV fault suspends balancing.

Note: BMW i3 CSC modules manage their own internal balancing. The BMS does not send balance commands in BMW i3 CAN mode.

# 8. Serial Console

## 8.1 Connection

Connect via USB-C. Open a serial monitor at 115200 baud, 8-N-1 with line endings enabled (LF or CRLF).

PlatformIO: click the monitor button or run:  pio device monitor

## 8.2 Short Commands (single key + Enter)

| Key | Action |
| --- | --- |
| H or ? | Print help menu with all commands |
| S | Sleep all CMU boards |
| W | Wake all CMU boards |
| C | Clear all faults on all boards |
| F | Re-scan for connected boards |
| R | Renumber boards from address 1 |
| B | Trigger one cell balancing cycle |
| p | Toggle pack summary printout every 3 seconds |
| d | Toggle pack detail printout every 3 seconds |

## 8.3 Configuration Commands (KEY=value + Enter)

All settings are saved to flash automatically after each valid command.

| Command | Range | Description |
| --- | --- | --- |
| VOLTLIMHI=4.20 | 0.0–6.0 V | Overvoltage threshold per cell |
| VOLTLIMLO=3.00 | 0.0–6.0 V | Undervoltage threshold per cell |
| IGNORECELL=2.50 | 0.0–3.0 V | Ignore cells below this voltage (use 2.5 for 4-cell packs) |
| TEMPLIMHI=55.0 | 0.0–100.0 °C | Over-temperature threshold |
| TEMPLIMLO=-10.0 | -20–120 °C | Under-temperature threshold |
| BALVOLT=4.10 | 0.0–6.0 V | Cell voltage target for balancing |
| BALHYST=0.04 | 0.0–1.0 V | Hysteresis below BALVOLT before balance stops |
| AUTOBAL=1 | 0 or 1 | Enable (1) / disable (0) automatic balancing |
| NUMCELLS=6 | 4, 5, or 6 | Global default cells per module (Tesla mode) |
| CELLCOUNT_n=6 | 1–20 (n), 0/4/5/6 | Per-module cell override (0 = use global default) |
| WIFI=1 | 0 or 1 | Enable WiFi on next boot |
| LOGLEVEL=1 | 0–4 | 0=Debug 1=Info 2=Warn 3=Error 4=Off |
| CMUTYPE=0 | 0 or 1 | CMU type: 0=Tesla UART, 1=BMW i3 CAN (reboot required) |
| CANINHIBIT=1 | 0 or 1 | Enable (1) CAN charger heartbeat balance inhibit |
| CHGID=0x305 | 0x001–0x7FF | CAN ID to watch for charger/inverter heartbeat |
| BATTERYID=1 | 1–14 | Battery ID transmitted in CAN frames |

# 9. Fault Codes

## 9.1 BQ76PL536A Fault Register (Tesla UART Mode)

| Bit | Meaning |
| --- | --- |
| 0x01 | Cell overvoltage (COV) |
| 0x02 | Cell undervoltage (CUV) — normal on 4-cell packs with unpopulated inputs; suppressed in fault display when IGNORECELL is set appropriately |
| 0x04 | CRC error in received UART packet |
| 0x08 | Power-on reset occurred |
| 0x10 | Test fault active |
| 0x20 | Internal registers inconsistent |

The on-display fault badge masks bit 0x02 (CUV) by default to avoid spurious red badges on packs with fewer than 6 cells per module. The raw fault register is always available via the serial console.

## 9.2 BMW i3 CAN Mode

BMW i3 CSC modules do not expose a fault register over CAN. Fault detection in i3 mode is limited to:

- Cell overvoltage: any cell reading above the OV setpoint

- Cell undervoltage: any cell reading below the UV setpoint

- Module timeout: no CAN frame received from a module for more than 3 seconds

# 10. Building and Flashing

## 10.1 Requirements

- Visual Studio Code with the PlatformIO IDE extension

- USB-C cable (data-capable)

## 10.2 Steps

1.  Open the TeslaBMS_M5Dial folder in VS Code.

2.  PlatformIO auto-detects platformio.ini and downloads the ESP32-S3 toolchain and libraries.

3.  Edit src/bms_config.h if you need to change BMS_NUM_SERIES or BMS_NUM_PARALLEL before first flash.

4.  Click Build (✓) to verify compilation.

5.  Click Upload (→) to flash. Hold the M5Dial power button if needed to enter download mode.

6.  Open the serial monitor at 115200 baud to view boot logs and enter commands.

Note: EEPROM version 0x17. If the device was previously flashed with v5.x firmware, the EEPROM version will not match and all settings will reset to defaults on first boot.

## 10.3 Library Dependencies (platformio.ini)

| Library | Version |
| --- | --- |
| m5stack/M5Unified | ^0.2.4 |
| m5stack/M5GFX | ^0.2.4 |
| lvgl/lvgl | ^8.4.0 |
| esphome/AsyncTCP-esphome | ^2.1.4 |
| esphome/ESPAsyncWebServer-esphome | ^3.3.0 |
| bblanchon/ArduinoJson | ^7.3.1 |